Start of oil pressure mode commissioning

Check if the wiring is correct

Restore factory parameters (Pr5.01=1)

Pr0.00= Motor power

Pr0.01= Motor voltage

Pr0.02= Motor current

Pr0.03= Motor frequency

Input motor nameplate parameters

Pr0.04= Motor speed

Motor parameter input completed

Pr0.26=1  Static tuning

Press the confirm key; the operation panel displays ARUN and flashes. Then press the RUN key; the drive enters tuning state. (After tuning is completed, the drive operation panel automatically returns to parameter display state)

Troubleshoot according to fault codes; contact the manufacturer if unresolved

If a fault is reported

Tuning steps

Start motor tuning

Tuning completed

If the rotation direction is incorrect

Modify parameter Pr0.18=0.0 or after power-off, swap any two phases of the motor, then power on and retune (Retuning is mandatory; direct operation is not allowed)

Check if the motor rotation direction is correct

Method

Set Pr0.12=5 (Set frequency, press the run key, check if the motor rotation direction matches the oil pump direction)

Rotation direction correct

Switch to oil pressure mode

P1.00 set to 2, servo drive enters oil pressure mode

Method

Set oil pressure parameters

PR1.01(Injection molding machine max speed)=XXX
PR1.02(System oil pressure)=XXX
PR1.03(Max oil pressure)=Corresponding to oil pressure sensor range
PR1.17(Injection molding machine max reverse speed)=XXX，generally default factory value